| Total pump time | Soil moisture | Decrease |
| --- | --- | --- |
| Dry | 712 | — |
| 1s | 689 | 23 |
| 2s | 663 | 26 |
| 3s | 640 | 23 |
| 4s | 615 | 25 |
| 5s | 591 | 24 |
| 6s | 567 | 24 |
| 7s | 542 | 25 |
| Average decrease per second |  | (23+26+23+25+24+24+25) ÷ 7 = 24.3 |

--Experimental data table

--CALIBRATED SERVER CODE;

import time

DECREASE_PER_SECOND = 24.

TARGET_MOISTURE     = 450

def smart_irrigate(sensor, relay):

current = sensor.moisture

print(f"Current moisture : {current}")

print(f"Target moisture  : {TARGET_MOISTURE}")

if current <= TARGET_MOISTURE:

print("Soil already moist enough. Skipping.")

return

moisture_gap = current - TARGET_MOISTURE

pump_seconds = moisture_gap / DECREASE_PER_SECOND

print(f"Pumping for      : {pump_seconds:.1f} seconds")

# Run the relay (virtual: simulate moisture drop)

relay.on()

time.sleep(pump_seconds)

relay.off()

sensor.moisture -= DECREASE_PER_SECOND * pump_seconds

print(f"New moisture     : {sensor.moisture:.0f}")