Sensor and Actuator for IoT Dev Kit

Sensor: DHT11 Temperature and Humidity Sensor

- What it does: It measures the air temperature and humidity around it. It is very popular for simple IoT projects like weather monitors or smart home devices.

- The electronics/hardware used inside: It has a thermistor (for temperature) and a capacitive humidity sensor inside. A small 8-bit chip reads the values and sends them out.

- Is it analog or digital: It is digital.

- What the units and range of inputs or measurements is: Temperature: °C (0 to 50 °C) Humidity: % RH (20% to 90%)

Actuator: SG90 Servo Motor

- What it does: It rotates to a exact angle (0° to 180°) so it can move arms, doors, or other mechanical parts in IoT projects.

- The electronics/hardware used inside: It contains a small DC motor, plastic gears, a potentiometer for position feedback, and a control circuit.

- Is it analog or digital: It is digital (controlled by PWM signal from the microcontroller).

- What the units and range of inputs or measurements is: Rotation angle: 0° to 180° Control signal: PWM pulse (1 ms = 0°, 1.5 ms = 90°, 2 ms = 180°)