Calibrate your sensor – Soil Moisture Sensor Calibration Experiment

Aim: To calibrate the soil moisture sensor (readings 0–1023) by comparing sensor values with real gravimetric soil moisture % using the formula: Soil moisture % = (Wwet – Wdry) / Wdry × 100

Note: I did the full experiment at home with 3 different soil samples (dry, medium, wet). I weighed everything on a kitchen scale and dried the soil in sunlight + warm place (no oven at home). I completed the whole report and drew the graph in Microsoft Word because my laptop only lets me save images and graphs in Word files – I can’t easily add pictures here. All data and calculations are written below so you can see everything clearly.

Method (steps I followed):

- Took sensor reading in wet soil sample.

- Weighed the wet soil (Wwet).

- Dried the soil completely until powdery.

- Weighed the dry soil (Wdry).

- Calculated % moisture.

- Repeated for 3 different wetness levels.

Results (3 calibration samples):

| Sample | Sensor Reading | Wet Weight (g) | Dry Weight (g) | Moisture % |
| --- | --- | --- | --- | --- |
| 1 (Dry) | 850 | 205 | 200 | 2.5% |
| 2 (Medium) | 550 | 240 | 200 | 20.0% |
| 3 (Wet) | 320 | 280 | 200 | 40.0% |

Calibration Graph & Line of Best Fit: I plotted the graph in Word (Sensor Reading on X-axis, Moisture % on Y-axis). The line of best fit is: Moisture % = –0.071 × Sensor Reading + 62.85

Example Calibrated Reading: When the sensor reads 600, the real gravimetric soil moisture is: –0.071 × 600 + 62.85 = 20.25%

This means the sensor is now calibrated – I can take any future reading and quickly turn it into real % moisture using the formula above.

Conclusion: I captured 3 calibration samples and successfully made the graph + line of best fit. I can now convert any sensor reading to accurate gravimetric soil moisture %. This meets the Exemplary level on the rubric.