| Aspect | Microcontroller (e.g., Arduino Uno, ESP32) | Single-Board Computer (e.g., Raspberry Pi 5) |
| --- | --- | --- |
| Definition | A small integrated circuit that houses the processor, memory, and I/O ports all on one chip | A fully integrated mini-computer on one board featuring a CPU, RAM, storage, and standard ports. |
| Cost | Very low (typically $1–$10) | Higher (typically $35–$100+) |
| Power Consumption | Very low (milliwatts) – perfect for devices that rely on batteries or have limited energy resources | Higher (several watts) – requires more stable power source |
| Processing Power | Limited (8–32 bit, MHz clock speeds, single or few simple cores) | High (multi-core GHz processors, comparable to older PCs) |
| Memory | Very small (KB to a few MB of RAM/flash) | Much larger (1 GB to 8+ GB RAM + external storage support) |
| Operating System | Usually bare-metal (no OS) or lightweight RTOS | Full-featured OS (Linux, etc.) |
| Real-Time Performance | Excellent – deterministic timing, no OS overhead | Adequate but not guaranteed due to OS scheduling |
| Physical Size | Tiny (chip or small board) | Larger (credit-card or larger size) |
| I/O & Peripherals | Direct access to GPIO, ADC, PWM, and timer – perfect for sensor/control devices. | It features GPIO but typically needs a HAT; it includes integrated USB, HDMI, Ethernet, and camera/display ports. |
| Programming | Simple embedded languages (C/C++, Arduino IDE) | Full desktop-style programming (Python, Java, etc.) + rich libraries |
| Typical Applications | Embedded systems, IoT sensors, motor control, wearables, and basic automation. | Media hubs, online servers, vision-enabled robotics, desktop-style initiatives, AI and machine learning projects |

Reasons to use a microcontroller over a single-board computer (2+ reasons):

- Lower power consumption and cost – Perfect for devices run by battery, Mass production, or long-term applications requiring energy and budget savings.

- Superior real-time performance – Microcontrollers provide accurate, low-latency control (such as motor timing or sensor sampling) independent of the operating system.

Reasons to use a single-board computer over a microcontroller (2+ reasons):

- Much higher processing power and memory – it can run complex tasks such as Image recognition, machine learning, web server management, and multitasking – these are tasks that microcontrollers find nearly impossible or too slow to handle.

- Easier software development and richer ecosystem – can run fully Linux operating system, use advance language, Leveraging thousands of easily accessible libraries and tools accelerates development and facilitates graphical interfaces along with sophisticated network services.